
     AMMONIUM FLUORIDE
Safety Data Sheet
Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       
      DUDLEY CORPORATION
           
       125 Kenyon Drive

       



         
       Lakewood, New Jersey 08701

Effective Date: January 1, 2025

1. PRODUCT ID: CAS No.: 12125-01-8  E-C No.:235-185-9 Weight:  37.04g/mol Formula: H4FN
2. HAZARDS ID: GHS:
Acute toxicity, Inhalation (Category 3)  Acute toxicity, Dermal (Category 3)

Acute toxicity, Oral (Category 3)



Pictogram: 



                   [image: image1.png]www onsafilines.com

Acute Toxicity




Signal Word:         DANGER
HAZARD STATEMENT(S): H301 + H311 + H331: Toxic if swallowed, in contact with skin or inhaled


PRECAUTIONARY STATEMENT(S): P261: Avoid breathing dust/spray/ fume/gas/ mist/ vapors. P264: Wash skin thoroughly after handling.   P271: Use only outdoors or in well-ventilated area. P405: Store locked up.  P270: Do not drink, eat, smoke, when using this product. P280: Wear protective gloves/eye & face protection. P301 + P310+P330: If swallowed: Call Poison Center or doctor/physician – Rinse mouth. P302 + P352+P312: If on Skin: Wash with plenty of soap & water. Call Poison Center or doctor/physician if unwell. P304 + P340: If inhaled: remove to fresh air and keep at rest in a position comfortable for breathing. P362: Take off contaminated clothing and wash before reuse. P403 + P233: Store in a well-ventilated place. Keep container tightly closed. P501: Dispose of contents/container to an approved waste disposal plant.

HMIS: Health hazard  2, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  2, Fire     0, Reactivity hazard   0

Potential Effects: Eye: Risk of serious damage to eyes.
Skin: Harmful in contact with skin. 

Ingestion: Harmful if swallowed. 

Inhalation: Harmful by inhalation. 
2.3: Other Hazards: none
3. COMPOSITION: 
Chemical Name



 CAS No.
 
Percent


Ammonium fluoride
              

  12125-01-8               98%                 
4. FIRST AID: Eyes: Flush eyes w/ water for 15 minutes, occasionally lifting the eyelids. Consult physician. 

Skin: flush skin with soap & water. Treatment with calcium gluconate paste. Consult physician.  Ingestion: Rinse mouth with water. Never give anything to an unconscious person. Drink milk, chewable calcium carbonate tablets or milk of magnesia. Consult Physician.  Inhalation: Remove to fresh air immediately. If not breathing, give artificial respiration. Consult physician.  Physician: Treat symptomatically and supportively.

5. FIRE FIGHTING:  Info:  Wear self-contained breathing apparatus in pressure-demand, MSHA/NIOSH, & full PPE.  Decomp products formed under fire: Nitrogen oxides (NOx), & hydrogen fluoride. Extinguishing: Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. Hazards: Nitrogen oxides, hydrogen fluoride
6. ACCIDENTAL RELEASE:  Info: Use proper PPE as indicated in Section 8. Spills/Leaks:  Vacuum or sweep up material and place into a suitable disposal container.  Clean up spills immediately, observing precautions in the Protective Equipment section. Avoid generating dusty conditions.  Provide ventilation.
7. HANDLING & STORAGE: Handling:  Wash thoroughly after handling.  Use with adequate ventilation.  Minimize dust generation and accumulation.  Avoid contact with eyes, skin, and clothing.  Avoid ingestion and inhalation.
Storage: Store in a tightly closed container.  Keep container in a cool, well-ventilated area away from incompatible substances. Hygroscopic do not store in glass.
8. EXPOSURE CONTROLS: Facilities storing or utilizing this material should be equipped with an eyewash facility and safety shower. Use adequate ventilation to keep airborne concentrations low.

Ammonium Fluoride:   CAS 12125-01-8   3.000mg/g ACGIH exposure limits.  2.5 mg/m3 TWA – OSHA & ACGIH
PPE: Eyes:  Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN 166. Skin:  Wear appropriate protective clothing to prevent skin exposure. 0.11mm thick. Clothing: Wear appropriate clothing (Lab coats, aprons) to prevent skin exposure.
Respirators: Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149.  Always use a NIOSH or European Standard EN 149 approved respirator when necessary.
9. PHYSICAL & CHEM: Appearance:  colorless crystalline powder. pH:  6 at 20°C Melting Pt:  100˚C(212˚F) Bulk Density:  ca.660 kg/m3
Decomp Temp:  100°C  Solubility:455g/l at 25˚C (77˚F)  Relative Density:  1.01 g/cm3 at 20˚C (68˚F)
10. STABILITY & REACTIVITY: Stability: Stable under normal storage & handling. Avoid:  Incompatibles, dust generation, excess heat. Hydroscopic reacts dangerously to glass. Incompatibilities:  Strong oxidizing agents. Haz Decomp:  See sec 5
11. TOXICOLOGICAL INFO:  RTECS:  BQ6300000 Acute Toxicity:  LD50 Intraperitoneal – rat – 31 mg/kg.  -  LD50 Oral Rat >200- <2,000 mg/kg  IARC, NTP, OSHA, ACGIH:    No component of this product present at levels greater than or equal to 0.l% is identified as probable, possible or confirmed human carcinogen Additional Info:   Stomach Irregularities, risk of serious eye damage.
12. ECOLOGICAL INFO: No info.
 
13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. 
RCRA P-Series: None listed. 
RCRA U-Series: None listed. 

14. TRANSPORT INFO: DOT:  UN 2505, Ammonium fluoride, 6.1, III,

RQ: 100 lbs.

IMDG:
UN 2505, AMMONIUM FLUORIDE, 6.1, III,
EMS-No: F-A, S-A

IATA:
UN 2505, Ammonium fluoride, 6.1, III,
15. REGULATORY INFO: SARA 311/312: Acute Health hazard
CERCLA Hazardous Substances and corresponding RQs None of the chemicals in this material have an RQ. 

SARA Section 302 Extremely Hazardous Substances None of the chemicals in this product have a TPQ.

SARA Codes:  CAS#12125-01-8:  Immediate, delayed.

Section 313 No chemicals are reportable under Section 313. 

OSHA Target organ Effect, Toxic by inhalation, Toxic by ingestion, Toxic by skin absorption. 

STATE CAS#12125-01-8:  Can be found on the following states right to know lists: 
New Jersey Right to Know Components: Ammonium Fluoride
12125-01-8      Rev date: 1993-04-24

Pennsylvania Right to Know Components: Ammonium Fluoride
12125-01-8      Rev date: 1993-04-24

Massachusetts Right to Know Components: Ammonium Fluoride
12125-01-8      Rev date: 1993-04-24

California Prop 65: No Significant Risk Level: None of the chemicals in this product are listed. 

WGK (Water Danger/Protection) CAS#12125-01-8: 1. 

Canada - DSL/NDSLCAS#12054-85-2 is listed on Canada’s DSL List
Canada – WHMIS:  Not available. CAS#12125-01-8 is not listed on Canada’s Ingredient Disclosure List. 

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

