
  ASTRA BLUE 6 GLL
Safety Data Sheet
Emergency Telephone Number: CHEMTREC 1-800-424-9300

For Information call:  DUDLEY 732-886-3100  
 DUDLEY CORPORATION        

          
     125 Kenyon Drive


          Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT ID: Syn: Basic Blue 140   CAS No.:   82864-57-1  Weight: 1,068.75G/MOL Formula: C47H52CuN14O6S3
2.HAZARDS ID: GHS Class: 
Skin Irritation ( Category 2) Eye Irritation ( Category 2A) 

Specific Target Organ Toxicity – Single Exposure ( Category 3)

Pictogram:



                 [image: image1.png]www onsafilines.com

Caution



  Warning

Hazard Statements: H315 – May cause skin irritation.


  H319 – Causes serious eye irritation.  H335 May cause respiratory irritation.
Precautionary Statements:  
P261 – Avoid breathing dust/ fume/ gas/ mist/ vapors/ spray. 


P305+P351+P338 – If in EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if 
present and easy to do. Continue rinsing. 

HMIS Classification: Health hazard 2, Flammability   0, Physical hazards   0




NFPA Rating: Health hazard 2, Fire     0, Reactivity hazard   0

Potential Effects: Eyes: Causes eye irritation.  Skin: Causes skin irritation. May be harmful if absorbed through skin.  Ingestion: May be harmful if swallowed.   Inhalation: May be harmful if inhaled. Causes respiratory tract irritation.
2.3: Other Hazards: none

3. COMPOSITION, INFO ON INGREDIENTS:
Chemical Name 


CAS No.

Percent


Hazardous
 Astra Blue 6GLL


82864-57-1
100%


      No


4. FIRST AID MEASURES: Eyes: Immediately flush eyes with plenty of water for at least 15 minutes. Consult a physician. 

Skin: Flush with soap and plenty of water. Consult a physician. Ingestion: NEVER give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician. Inhalation: Remove from exposure to fresh air immediately. If not breathing, give artificial respiration. Consult physician. Physician: Treat symptomatically & supportively.

5. FIRE FIGHTING: Info: Wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH, & full PPE.  Dusts at sufficient concentration can form explosive mixtures with air.  Irritating & highly toxic gases may be generated by thermal decomp or combustion. Extinguishing: Use water spray, dry chem, Carbon dioxide, or alcohol-resistant foam. Flash Pt: N/A Auto-ignition Temp: N/A Explosion Limits: N/A Hazards:  Carbon oxides, nitrogen oxides (NOx), Sulphur oxides, Copper oxides

6. ACCIDENTAL RELEASE: Info: Use proper PPE as indicated in Section 8.  Spills/Leaks: Vacuum or sweep up material and place into a suitable disposal container. Clean up spills immediately, observing precautions in the PPE section. Avoid generating dusty conditions. Provide ventilation. Do not let product enter drains.
7. HANDLING & STORAGE: Handling: Wash thoroughly after handling. Use with adequate ventilation. Minimize dust generation & accumulation.  Avoid contact with eyes, skin, and clothing. Keep container tightly closed. Avoid ingestion and inhalation.  Storage: Store in a tightly closed container. Store in a cool, dry, well-ventilated area away from incompatibles. 

8. EXPOSURE CONTROLS: Facilities storing or utilizing this material should be equipped with an eyewash facility and safety shower. Use adequate ventilation to keep concentrations low.

Chemical 



ACGIH

NIOSH

OSHA – Final/Vacated PEL’s
[N,N’N”-tris[3-(dimethylamino)propyl]-29H,
None listed
TWA 1mg/m3
None listed

31H-phthalocyaninetrisulphonamidato(2-)-N29,N30,N31,N32] copper triacetate


PPE: Eyes: Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. Safety glasses and side shields. Skin: Wear appropriate protective gloves to prevent skin exposure, and proper removal technique. Dispose of contaminated gloves after use.  Clothing: Wear appropriate protective clothing to prevent skin exposure. Respirators: Follow the OSHA respirator regulations found in 29CFR 1910.134 or European Standard EN 149. Always use a NIOSH or European Standard EN 143 approved respirator when necessary. P95 (US)
9. PHYSICAL & CHEM: Appearance: Dark blue powder

10. STABILITY & REACTIVITY: Stability: Stable at room temperature in closed containers under normal storage 
and handling conditions. Avoid: High temperatures, dust generation. Incompatibilities:   Strong oxidizing agents.

Hazardous Decomp Products: see sect 5
11. TOXICOLOGICAL INFO: RTECS:  unlisted.  LD50/LC50: N/A. Carcinogenicity: Not listed ACGIH, IARC, NTP, or OSHA. 

12. ECOLOGICAL INFO: No data.

13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified 
as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, 
waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. 
RCRA P-Series: None listed. 
RCRA U-Series: None listed. 

14. TRANSPORT INFO: Not regulated.

15. REGULATORY INFO: SARA 311/312 Hazards:  Acute Health Hazard
SARA Section 302 Extremely Hazardous Substances None of the chemicals in this product have a TPQ/RQ
Section 313: [N,N’N”-tris[3-(dimethylamino)propyl]-29H,31H-phthalocyaninetrisulphonamidato(2-)-N29,N30,N31,N32] 
copper triacetate 
CAS-No.: 82864-57-1
Revision Date 2007-07-01
STATE CAS# 82864-57-1 is not present on state lists from CA, MA, MN, or FL. 

California No Significant Risk Level: None of the chemicals in this product are listed. 

PA Right To Know Components:  [N,N’N”-tris[3-(dimethylamino)propyl]-29H,31H-phthalocyaninetrisulphonamidato (2-)-N29,N30,N31,N32] copper triacetate CAS-No.: 82864-57-1
Revision Date 2007-07-01
NJ Right To Know Components: [N,N’N”-tris[3-(dimethylamino)propyl]-29H,31H-phthalocyaninetrisulphonamidato
(2-)-N29,N30,N31,N32] copper triacetate  
CAS-No.: 82864-57-1
Revision Date 2007-07-01
EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives 

WGK (Water Danger/Protection) CAS# 82864-57-1: 1 Canada - DSL/NDSL is listed on Canada’s DSL List. 

Canada – WHMIS This product has a WHMIS classification of NOT CONTROLLED.
CAS# 82864-57-1 is not listed on Canada’s Ingredient Disclosure List. 

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

