
     FERRIC CHLORIDE HEXAHYDRATE

Safety Data Sheet

Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       
      DUDLEY CORPORATION
           
       125 Kenyon Drive

       



         
       Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT ID:  Syn:  Iron (III) chloride hexahydrate              CAS No.: 10025-77-1 E-C No.:231-729-4

                              Weight:  270.30g/mol 

                   Formula: Cl3Fe.6H2O
2. HAZARDS ID: GHS Classification:  Acute toxicity, Oral (Category 4)
    Skin irritation (Category 2)





Serious Eye damage (Category 1)

 Pictogram: 



[image: image1.png]Corrosive




[image: image2.emf]
Signal Word:         DANGER
HAZARD STATEMENT(S): H302: Harmful if swallowed.  H318: Causes serious eye damage. H315: Causes skin irritation
 
PRECAUTIONARY STATEMENTS): P264: Wash skin thoroughly after handling. P270: Do not drink, eat, smoke, when using this product.P280: Wear protective gloves/eye & face protection. P301 + P312 + P330: If swallowed: Call Poison Center or doctor/physician if you feel unwell, rinse mouth. P302 + P352: If on skin: Wash with plenty of soap & water.  P305 + P351 + P338: If in Eyes: Rinse cautiously with water for several minutes. P310: Remove contact lenses, if present & easy to do so. Continue rinsing. P332 + P313: If skin irritation occurs: consult physician. P390: Absorb spillage to prevent material damage. P362: Take off contaminated clothing and wash before reuse. P406: Store in corrosive resistant stainless steel container with 
a resistant inner liner. P501: Dispose of contents/container to an approved waste disposal plant.

HMIS Classification: Health hazard  2, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  2, Fire     0, Reactivity hazard   0
2.3: Other Hazards: none
3. COMPOSITION / INFO ON INGREDIENTS: 
Chemical Name


 CAS No.
 
Percent


Iron trichloride hexahydrate                     10025-77-1              >98%                  

4. FIRST AID: Eyes: Rinse immediately with plenty of water. Skin: Rinse with plenty of soap & water. Ingestion: Do not induce vomiting without medical advice. Rinse with water. Inhalation: Move to fresh air.  Physician: Treat symptomatically & supportively. 

5. FIRE FIGHTING: Info: Wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH & full PPE. Irritating & highly toxic gases may be generated by thermal decomp/combustion. Hazards:  Hydrogen chloride gas, Iron oxides. Extinguishing:  Water Spray, Alcohol-resistant foam, dry chemical or carbon dioxide. 
6. ACCIDENTAL RELEASE: Info: Use proper PPE as indicated in Section 8. Spills: Vacuum or sweep up material and place into a suitable disposal container. Clean up spills immediately, observing precautions in the PPE section. Avoid generating dusty conditions. Provide ventilation. Do not let this product enter drains. Avoid breathing vapors, dust, mist, or gas. 
7. HANDLING & STORAGE: Handling: Wash thoroughly after handling. Use with adequate ventilation. Minimize dust generation and accumulation.  Avoid contact with eyes, skin, and clothing. Keep container tightly closed. Avoid ingestion and inhalation. Storage: Store in a tightly closed container. Store in a cool, dry, well-ventilated area away from incompatibles.  Corrosives area.  Store protected from moisture and light. Store under inert gas. Hydroscopic.
8. EXPOSURE CONTROLS, PPE:  Controls: Facilities storing or utilizing this material should be equipped with an eyewash facility and safety shower. Use adequate ventilation to keep airborne concentrations low.

Chemical Name


ACGIH          

NIOSH          
            OSHA - Final PEL’s
Iron Trichloride 6 HY

1mg/m3 TWA 

1mg/m3 TWA 
           1 mg/m3 TWA 

(Threshold,

(recommended
              (Limits for Air 
Limit values)

exposure limits) 
              contaminants 1910.1000)

PPE: Eyes: Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. Skin: Wear appropriate protective gloves to prevent skin exposure. 0.11mm thick nitrile. Clothing: Wear appropriate protective clothing to prevent skin exposure. 
Respirators: Follow the OSHA respirator regulations found in 29CFR 1910.134 or European Standard EN 149. Always use a NIOSH or European Standard EN 149 approved respirator when necessary. Hygiene measures: Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.
9. PHYSICAL & CHEM: Yellow powder.   Melting Pt: 99˚F, 37˚C  

Vapor Pressure: 1hPa (1mmHg) at 194˚C.  Boiling Pt: 280-285˚C (536-545˚F),
Relative Density: 1.820g/cm3

10. STABILITY & REACTIVITY: Stability: Stable at room temperature in closed containers under normal storage and handling conditions. May decompose on exposure to moist air or water.  Moisture sensitive. Avoid: Moisture, heating to decomposition. Incompatibilities: Strong oxidizing agents, forms shock-sensitive mixtures with certain other materials (sodium & potassium oxides.) Haz Decomp: see section 5
11. TOXICOLOGICAL INFO: RTECS: NO5425000  LD50/LC50: Oral, rat: 316 mg/kg; Dermal >2,000 mg/kg
Carcinogenicity: Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA.

12. ECOLOGICAL INFO: Environmental Fate: Bluegill Sunfish – 20.3mg/l – 96h.
Water flea – 9.6mg/l – 48h
13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste.US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. 

RCRA P-Series: None listed. RCRA U-Series: None listed. 

14. TRANSPORT:  Not regulated.
SARA Section 302 Extremely Hazardous Substances None of the chemicals in this product have a TPQ/RQ.

Section 313 No chemicals are reportable under Section 313. 

STATE CAS# 10025-77-1 can be found on the following state right to know lists: 

California 65 No Significant Risk Level: None of the chemicals in this product are listed. 

New Jersey Right to Know Components:  Iron trichloride hexahydrate   CAS#10025-77-1 Revision:1993-04-24

Pennsylvania Right to Know Components: Iron trichloride hexahydrate CAS#10025-77-1 Revision:1993-04-24

Massachusetts Right to Know Components: Iron trichloride hexahydrateCAS#10025-77-1 Revision:1993-04-24

EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives 

Hazardous Symbols: C    WGK (Water Danger/Protection) CAS# 10025-77-1: No information available.

Canada - DSL/NDSL CAS# 10025-77-1 is not listed on Canada’s DSL List. 

Canada – WHMIS This product has a WHMIS classification of E. 

CAS# 10025-77-1   is listed on Canada’s Ingredient Disclosure List. 

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this information.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

