
INDIGO
Safety Data Sheet

Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       
      DUDLEY CORPORATION
           
   125 Kenyon Drive

       



         
   Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT: Syn:  Indigo blue, Indigoton 
CAS No.: 482-89-3 E-C No.:207-586-9 Weight:  262.26g/mol 
Formula: C16H10N2O2
2. HAZARDS: GHS:
Specific target organ toxicity – single exposure (Category 3) respiratory system.
Skin irritation (Category 2), Eye irritation (Category 2A)
 Pictogram: 




[image: image1.png]www onsafilines.com

Caution







Signal Word:         WARNING

HAZARD STATEMENT: H315: Causes skin irritation. P319: Causes serious eye irritation. H335: May cause respiratory irritation.
PRECAUTIONARY STATEMENT: P261: Avoid breathing dust/spray/ fume/gas/ mist/ vapors. P264: Wash skin thoroughly after handling. P271: Use only outdoors or in a well-ventilated area. P280: Wear PPE P302 + P352: If on Skin: wash with plenty of water & soap P304+P340+P312: If Inhaled: remove person to fresh air & keep comfortable. P305+P351+P338: If in eyes: rinse cautiously with water for several minutes. Remove contacts if easy to do so. P332 + P337 + P313: If skin & eye irritation occurs get medical attention. P362: Take off contaminated clothing and wash before reuse. P403+P233: Store in a well-ventilated place. Keep container tightly closed. P405: Store locked up. P501: Dispose of contents/container to an approved waste disposal plant.

HMIS Class: Health hazard 0, Flammability 0,  Physical hazards 0




NFPA Rating: Health hazard 0, Fire  0, Reactivity hazard   0
2.3: Other Hazards: none
3. COMPOSITION / INFO ON INGREDIENTS: 
Chemical 


 CAS No.
 
Percent


Indigo


                482-89-3               100%                  

4. FIRST AID: Eyes: rinse immediately for 15 min. with plenty of water and seek medical advice. Skin: Rinse with plenty of soap & water and seek medical advice. Ingestion: Do not induce vomiting without medical advice. Rinse with water. Consult physician. Inhalation: Move to fresh air.  Consult a physician. Physician: Treat symptomatically & supportively. 

5. FIRE FIGHTING: Info: wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH & full PPE. Irritating & toxic gases may be generated by thermal decomp/combustion. Hazards: Carbon & Nitrogen oxides. Extinguishing: use water, dry chem, carbon dioxide, or alcohol-resistant foam. Flash Pt: >220˚C(>428˚F) Auto-ignition Temp: >300˚C(>572˚F) Explosion Limits: N/a. 
 

6. ACCIDENTAL RELEASE: Info: Use proper PPE as indicated in Section 8. Spills/Leaks: Evacuate personnel to safe areas. Sweep up, then place into a suitable container for disposal. Avoid generating/breathing dusty conditions. Provide ventilation.  Do not let material enter Drains.
7. HANDLING & STORAGE: Handling: Wash thoroughly after handling. Remove contaminated clothing and wash before reuse. Use with adequate ventilation. Avoid contact with eyes, skin, and clothing. Avoid formation of dust and aerosols. Avoid ingestion and inhalation. Store protected from light. Storage: Keep containers tightly closed. Store protected form light. Store in a cool, dry area away from incompatible substances. 

8. EXPOSURE: Facilities storing or utilizing this material should be equipped with an eyewash facility and a safety shower. Use adequate ventilation to keep airborne concentrations low. 

Chemical 
                        ACGIH                       NIOSH                OSHA - Final PELs 

C.I. 73000
                       None listed                    None listed           None listed

PPE: Eyes: Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. Skin: Wear appropriate protective clothing to prevent skin exposure. 0.11mm thick nitrile Clothing: Wear appropriate protective clothing to prevent skin exposure. 

Respirators: A respiratory protection program that meets OSHA’s 29 CFR 1910.134 and ANSI Z88.2 requirements or European Standard EN 149 must be followed whenever workplace conditions warrant a respirator’s use. 

9. PHYSICAL & CHEM: Appearance: Dark purple powder. Relative Density: 1.35 g/cm3 at 20˚C Melting Pt: >300˚C 
10. STABILITY & REACTIVITY: Stability: Stable under ordinary conditions of use & storage. Light sensitive.

Avoid: N/A. Incompatibilities: Strong Oxidizing agents. Haz Decomp: see section 5
11. TOXICOLOGICAL: RTECS: DU2988400
LD50/LC50: Oral, Rat male = 6,400 mg/kg; 

Carcinogenicity: Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA. 

12. ECOLOGICAL: Fate: Biodegradability: aerobic 28d, result 0% Toxicity:  Toxicity to Fish – LC50 Golden Orfe - >10,000 mg/l-96h  Static test EC50-water flea- >500 mg/l -48h   EC50 Green algae- >100 mg/l-72h

13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. RCRA P-Series: None listed.    RCRA U-Series: None listed. 

14. TRANSPORT REGULATIONS: Not regulated
15. REGULATORY:  SARA 311/312: None
Section 302 (TPQ/RQ) None of the chemicals in this product have a TPQ/RQ. 

Section 313 No chemicals are reportable under Section 313. STATE is not present on state lists from CA, MN, MA, FL. 

PA & NJ Right to Know:
Indigo  CAS#482-89-3

California Prop.65: None of the chemicals are known to cause cancer, birth defects, or any other reproductive harm.

EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives 

Hazard Symbols: XN WGK (Water Danger/Protection) CAS# 482-89-3: 1 

Canada - DSL/NDSL CAS# 482-89-3 is listed on Canada’s DSL List. 

Canada - WHMIS This product has a WHMIS classification of Not controlled. 

16. DISCLAIMER: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

