
           
METHYLENE BLUE ANHYDROUS
Safety Data Sheet
Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       

      DUDLEY CORPORATION
        125 Kenyon Drive

       



         
        Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT: Syn:  Basic Blue 9, Tetramethylthionine chloride CAS No.: 61-73-4 E-C No.: 200-515-2

Weight:  373.90g/mol 
Formula: C16H18CIN3S

2. HAZARDS: GHS Class: Acute toxicity, Oral (Category 4) 
 Pictogram: 



                     [image: image1.emf]
Signal Word:         WARNING

HAZARD STATEMENT: H302: Harmful if swallowed


PRECAUTIONARY STATEMENT: P264: Wash skin thoroughly after handling. P270: Do not drink, eat, smoke, when using this product. P301 + P312 +P330: If swallowed: Call Poison Center or doctor/physician if you feel unwell. Rinse mouth.

P501: Dispose of contents/container to an approved waste disposal plant.

HMIS Class: Health hazard  2, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  2, Fire     0, Reactivity hazard   0

2.3: Other Hazards: none

3. COMPOSITION / INFO: 
Chemical 



 CAS No.
 
Percent


Methylthioninium chloride

61-73-4 

90-100%                  

4. FIRST AID: Eyes: Rinse with water. Skin: Rinse with soap & water. Ingestion: Do not induce vomiting without medical advice.  Inhalation: Move to fresh air.  Physician: Treat symptomatically & supportively. 

5. FIRE FIGHTING: Info: Wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH & full PPE.  Irritating & toxic gases may be generated by thermal decomp/combustion.  Hazard:  Carbon, Nitrogen, Sulphur oxides & hydrogen chloride gas. Extinguishing: Water spray, carbon dioxide, dry chem or alcohol-resistant foam. Flash Pt:  N/A. Auto-ignition Temp:  N/A. Explosion Limits:  N/A. Fire: Methylene Blue does not burn. Not considered a fire hazard. 

6. ACCIDENTAL RELEASE: Info: Use proper PPE as indicated in Section 8. Spills: Ventilate area of leak or spill. Pick up and place in a suitable container for reclamation or disposal, using a method that does not generate dust.   Clean up spills immediately, observing precautions in the PPE section.

7. HANDLING & STORAGE: Handling: Wash thoroughly after handling. Remove contaminated clothing and wash before reuse. Avoid contact with eyes, skin, and clothing.  Avoid dust formation and control ignition sources.    

Storage:  Store in a tightly closed container.  Store in a cool, dry ventilated area away from incompatibles.

8. EXPOSURE: Facilities storing or utilizing this material should be equipped with and eyewash facility and safety shower.  Use adequate ventilation to keep airborne concentrations low.
Chemical 


ACGIH

NIOSH


OSHA-Final / Vacated PEL’s

Methylene Blue


None listed
None listed

None listed

PPE: Eyes:  Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN 166. Skin:  Wear appropriate protective gloves to prevent skin exposure. 0.11mm thick nitrile. Clothing:  Wear appropriate protective clothing to prevent skin exposure.

Respirators: Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149.  Always use a NIOSH or European Standard EN 149 approved respirator when necessary.

9. PHYSICAL & CHEM: Dark green powder Solubility: Soluble Melting Pt: 190˚C (374˚F). 
10. STABILITY & REACTIVITY: Stability: Stable under ordinary conditions of use & storage. Avoid:  Heat, flame, ignition sources, dusting and incompatibilities. Incompatibilities:  Strong oxidizing agents. Haz Decomp: see section 5

11. TOXICOLOGICAL INFO: RTEC:  SP5740000
LD50/LC50:  Oral Rat 1,180 mg/kg
Carcinogenicity: Not listed by ACGIH, IARC, NIOSH, NTP or OSHA.

12. ECOLOGICAL INFO: No information.
13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste.  US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3.  Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. RCRA P-Series:  None listed RCRA U-Series:  None listed.

14. TRANSPORT REGULATIONS:  Not regulated.  

15. REGULATORY INFO: SARA 311/312: Acute Health Hazard, Chronic Health Hazard.

SARA Section 302 Extremely Haz: None of the chemicals in this product have a TPQ/RQ.

Section 313:  No chemicals are reportable under Section 313

STATE is not present on state lists from CA, MN, MA, FL. 

PA & NJ Right to Know: Methylthioninium chloride  CAS#61-73-4
California Prop.65: None of the chemicals are known to cause cancer, birth defects, or any other reproductive harm.

EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives

Hazardous Symbols:  XN WGK (Water Danger/Protection) CAS#61-73-4:  Not available.

Canada-DSL/NDSLCAS#61-73-4  is not listed on Canada’s DSL List

CAS#61-73-4 is not listed on Canada’s NDSL List.  Canada – WHMIS: CAS#61-73-4 WHMIS is not available.

CAS#61-73-4  is not listed on Canada’s Ingredient Disclosure List.

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

