
1-NAPHTHOL

Safety Data Sheet

Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       
      DUDLEY CORPORATION
           
       125 Kenyon Drive

       



         
       Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT ID: Syn: 1-Hydroxynaphthalene CAS No.: 90-15-3 E-C No.:201-969-4 Weight: 144.17g/mol  
                                  Formula: C10H8O

2. HAZARDS ID: GHS: Acute aquatic toxicity (Cat 1) Skin irritation (Cat 2) Acute toxicity, Dermal (Cat 3) Acute toxicity, Oral (Cat 4) Chronic aquatic toxicity (Category 3) Serious Eye damage (Category 1) Specific target organ toxicity - single exposure (Category 3) Respiratory system, Specific target organ toxicity - single exposure (Category 2) Kidney Skin sensitization (sub-cat 1A)
Pictogram: 


[image: image1.png]www onsafilines.com

Acute Toxicity




[image: image2.png]Corrosive




[image: image3.emf]     [image: image4.png]www onsafilines.com

Hazardous to the Aquatic
Environment



   [image: image5.png]www onsafilines.com

Longer Term Health Hazards




Signal Word:         DANGER
HAZARD STATEMENT(S): H302: Harmful if swallowed H311: Toxic in contact with skin. H318: Causes serious eye damage. H315: Causes skin irritation H335: May cause respiratory irritation H400: Very toxic to aquatic life. H412: Harmful to aquatic life with long lasting effects 


PRECAUTIONARY STATEMENT(S): P260: Do Not breathe dust/spray/ fume/gas/ mist/ vapors. P264: wash skin thoroughly after handling P270: Do not drink, eat, smoke, when using this product. P271: Use only outdoors or in well-ventilated area. P273: Avoid release into the environment P272: Contaminated work clothing must not be allowed out of the workplace. P405: Store locked up. P280: Wear protective gloves/eye & face protection
P362: Take off contaminated clothing and wash before reuse. P301 + P312 + P330: If Swallowed: Call Poison Center or doctor/physician if you feel unwell. Rinse Mouth
P302 + P352 + P312: If on skin gently wash with plenty of soap & water. Call a Poison center/ doctor/physician if you feel unwell
P304 + P340 + P312: If inhaled: remove to fresh air and keep at rest in a position comfortable for breathing. Call a Poison center/ doctor/physician if you feel unwell.  P391: Collect spillage  P305 + P351 + P338 + P310: If in Eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present & easy to do so. Continue Rinsing. Immediately call a Poison center/ doctor/physician if you feel unwell. P308 + P311: If exposed or concerned: Call Poison Center / Doctor. 
P332 + P313: If skin irritation occurs: consult physician   P403 + P233: Store in a well-ventilated place. Keep container tightly closed. P501: Dispose of contents/container to an approved waste disposal plant.

HMIS Classification: Health hazard  3, Flammability   1,  Physical hazards   0




NFPA Rating: Health hazard  3, Fire     1, Reactivity hazard   0

2.3: Other Hazards: none

3. COMPOSITION / INFO: 
Chemical Name


 CAS No.
 
Percent


1-Naphthol               

90-15-3              
100%                  

4. FIRST AID: Eyes: rinse immediately with plenty of water for 15 min.  Skin: Rinse with plenty of soap & water 

Ingestion: Do not induce vomiting without medical advice. Rinse with water. 

Inhalation: Move to fresh air.  Physician: Treat symptomatically and supportively. 

5. FIRE FIGHTING: Not considered to be a fire/explosion hazard under normal conditions. Wear self-contained breathing apparatus & PPE to prevent contact with skin & eyes. Special Haz: Carbon oxides Extinguishing: Water Spray, Carbon Dioxide, Dry Chem Powder or alcohol-resistant foam.
6. ACCIDENTAL RELEASE: Info: Wear self-contained breathing apparatus, rubber boots and heavy rubber gloves Spills/Leaks:  Sweep up, place in a bag and hold for waste disposal.  Avoid raising dust.  Ventilate area and wash spill site after material pickup is complete. 

7. HANDLING & STORAGE: Handling: Do not breathe dust. Avoid contact w/ eyes, skin and clothing. Storage: Store in a tightly closed container.  Protect container from physical damage.  Store in a cool, dry ventilated area from sources of heat or ignition.

8. EXPOSURE CONTROLS, PPE: Controls: Safety shower and eye bath. Mechanical exhaust required.
Chemical



ACGIH

NIOSH

OSHA-Final PELs

1-Naphthol



None listed
None listed
None listed
PPE: Eyes: Wear chemical safety goggles. Skin: Rubber gloves, and Rubber boots.
0.6mm latex / 0.3mm butyl-rubber
Clothing: Appropriate protective clothing. Respirators: Wear NIOSH/MSHA-Approved respirators.
9.PHYSICAL & CHEM:  Appearance: Solid Boiling Pt: 278-280˚C Melting Pt: 94-97˚C   Vapor Pressure: 2.3 hPa (1.7 mmHg) at 100˚C (212˚F) Lower Explosion limit: 0.8%(V) Upper Explosion limit: 5% (V)

Partition coefficient:  log Pow 2.85 Flash pt: 125˚C – closed cup Relative Density: 1.27 at 20°C Surface Tension: ca. 0.06 n/m at 1 g/l at 20.3°C OECD Test Guideline 115
10. STABILITY & REACTIVITY: Stability: Stable. Haz Decomp: see section 5 Incompatibilities: Strong Oxidizing 
Agents. Strong bases.
11. TOXICOLOGICAL INFO:  Oral LD50 Oral rat – 1,000-2,000 mg/kg
Dermal LD50 – rabbit -880 mg/kg
Skin: rabbit-severe skin irritation -24h Reproductive:   toxicity mouse – subcutaneous – effects on litter size.
Symptoms of Exposure: Cough, shortness of breath, headache, nausea, vomiting, to the best of our knowledge.

RTECS:  QL2800000   Carcinogenicity: IARC, ACGIH, NTP, OSHA – no component over 0.1% possible carcinogen.
12. ECOLOGICAL INFO: Toxic to aquatic life.
Toxicity to Fish:  LC50 – Pimephales promelas (Bluegill) – 0.76 mg/l – 96h  (Water Flea) – 2.51 mg/l – 48h
ErC50: Algae >2.18 mg/l

13. DISPOSAL CONSIDERATIONS: Chemical waster generators must determine whether a discarded chemical is 
classified as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. 
Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate 
classification. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with 
an afterburner and scrubber.  Observe all Federal, State and local Environmental Regulations.

RCRA P-Series: None listed
RCRA U-Series: None listed
14. TRANSPORT INFO: DOT: UN 2811, Toxic solids, organic, n.o.s. (1-Naphthol), 6.1, III, ERG 154   
IMDG: UN 2811, Toxic solid, organic, n.o.s. (1-Naphthol), 6.1, III,   

IATA: UN 2811, Toxic solid, organic, n.o.s. (1-Naphthol), 6.1, III, P.I.   
15. REGULATORY INFO: SARA 311/312:   Acute Health Hazard, Chronic Health Hazard.

SARA Section 302 Extremely Hazardous Substances None of the chemicals in this product have a TPQ.

Section 313 No chemicals are reportable under Section 313. 

STATE CAS# 83833-14-1 is not present on state lists from CA, MA, FL. 

NJ & PA Right to Know components:
1-Naphthol
CAS#90-15-3

California Prop 65:  No Significant Risk Level: None of the chemicals in this product are listed. 

EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives 

WGK (Water Danger/Protection) CAS#90-15-3:  No information available. 

Canada - DSL/NDSLCAS# 90-15-3 is not listed on Canada’s DSL List. 

Canada – WHMIS:  This product has been classified in accordance with the hazard criteria of the CPR, and the MSDS contains all the information required by the CPR CAS# 90-15-3 -1 is not listed on Canada’s Ingredient Disclosure List. 

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   
Page 2 of 2

