
     POTASSIUM OXALATE MONOHYDRATE
Safety Data Sheet

Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       

      DUDLEY CORPORATION
           
     125 Kenyon Drive

       



         
     Lakewood, New Jersey 08701

Effective Date: January 1, 2025


1. PRODUCT: Syn: Oxalic acid potassium salt, Ethanedioic acid 
CAS No.: 6487-48-5  E-C No.:209-506-8  Weight:  184.23g/mol 
Formula: C2K2O4.H2O
2. HAZARDS: GHS:  Acute toxicity, Dermal (Cat 4) Acute toxicity, Oral (Cat 4) Eye irritation (Cat 2A), Acute aquatic hazard (Cat 3)
 Pictogram: 



                        [image: image1.emf]
Signal Word:         WARNING

HAZARD STATEMENT(S): H302 + H312: Harmful if swallowed, in contact with skin. H319: Causes serious eye irritation H402: Harmful to aquatic life. 
PRECAUTIONARY STATEMENT(S) : P264: Wash skin thoroughly after handling. P270: Do not drink, eat, smoke, when using this product. P273: Avoid release to the environment. P280: Wear protective gloves/eye & face protection. P301 + P312 + P330: If Swallowed: Call Poison Center or doctor/physician if you feel unwell, Rinse mouth. P302 +P352 + P312: IF On Skin: Wash Plenty of water. Call a Poison Center / Doctor. P305 + P351 + P338: If In eyes: Rinse Cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337 + P313: If eye irritation persists. Get medical advice / attention. P363: Wash contaminated clothing before reuse. P501: Dispose of contents/container to an approved waste disposal plant.

HMIS: Health hazard  2, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  1, Fire     0, Reactivity hazard   0

Potential Effects: Eye: Risk of serious damage to eyes.

Skin: Harmful in contact with skin. 

Ingestion: Harmful if swallowed. 

Inhalation: Harmful by inhalation. 
2.3: Other Hazards: none
3. COMPOSITION / INFO: 

Chemical 


CAS No.

Percent

Hazardous
Potassium Oxalate Monohydrate ACS                   6487-48-5
90-100%                    No
4. FIRST AID:  Eyes: Flush eyes with plenty of water as precaution.  Skin:  Flush with soap and water. Ingestion: NEVER give anything by mouth to an unconscious person.  Rinse mouth with water. Inhalation: Remove to fresh air. If not breathing, give artificial respiration.

5. FIRE FIGHTING: Info: wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH & full PPE. irritating & highly toxic gases may be generated by thermal decomp/combustion. Hazards: Carbon & Potassium oxides. Extinguishing:  Water spray, dry chem, carbon dioxide or alcohol-resistant foam.
6. ACCIDENTAL RELEASE: Info:  Use proper PPE as indicated in Section 8.  Shut off all sources of ignition. Evacuate area. Avoid breathing in dust.  Spills: Vacuum or sweep up material & place into a suitable disposal container.  Use non-sparking tools and equipment.  Wash spill site after material pick-up is complete. Do not let product enter drains. 

7. HANDLING & STORAGE: Handling:  Wash after handling.  Use with adequate ventilation. Avoid contact in eyes, on skin, on clothing.  Avoid formation of dust and aerosols. Storage:  Keep tightly closed. Store in a cool dry, well ventilated area away from incompatibles.  Hygroscopic.
8. EXPOSURE:  Facilities storing or utilizing this material should be equipped with an eyewash facility and safety shower.  Use adequate ventilation to keep airborne concentrations lows.
Chemical 


ACGIH

NIOSH

OSHA – Final PEL’s
Potassium Oxalate Monohydrate 
ACS None listed
None listed
None listed

PPE: Eyes:  Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN 166. Safety goggles. Skin:  Wear appropriate protective gloves to prevent skin exposure. 0.11mm thick nitrile. Clothing: Wear appropriate protective clothing, including boots, lab coat, apron or coveralls, to prevent contract. Respirators:  Follow the OSHA respirator regulation found in 29 CFR 1910.134 or European Standard EN 149.  Always use a NIOSH or European Standard EN 149 approved respirators when necessary. Hygiene: Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday. 
9. PHYSICAL & CHEM: White / beige powder Density: 2.127  g/cm3  pH: 7.0-8.5 at 50g/l at 25˚C  

10. STABILITY & REACTIVITY: Stability: Stable under ordinary conditions of use & storage. Avoid:  Avoid Moisture. 
Haz Decomp.: see section 5 Incompatibilities: Halogens, Ammonia, cyanides, Heavy metals.
11. TOXICOLOGICAL:  RTECS:  Not available.
LD50: Oral Rat 375mg/kg  Dermal 1,100.1 mg/kg 
12. ECOLOGICAL: Toxicity: EC50: Water flea >33mg/l – 48h, ErC50: Green Algae 78mg/l-72h, EC10: Green Algae 78mg/l-72h
13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste.  US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3.  Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. Contact a licensed professional waste disposal service to dispose of this material. 
RCRA P-Series:  None listed.
RCRA U-Series:  None listed.
14. TRANSPORT REGULATIONS: Not regulated
15.REGULATORY: Section 311/312:  Acute & Chronic Health Hazard.
SARA Section 302 Extremely Hazardous Substances:
None of the chemicals in this product have a TPQ/RQ
Section 313:  No chemicals are reportable under Section 313.

STATECAS# 6487-48-5 is not present on state lists from CA, PA, MA, MN, FL, or NJ.

Pennsylvania Right To Know Components: Dipotassium oxalate, CAS# 6487-48-5
New Jersey Right To Know Components: Dipotassium oxalate, CAS# 6487-48-5
California Prop. 65 Components:  This product does not contain any chemicals know to the State of California to cause cancer, birth defects, or any other reproductive harm. 
16. DISCLAIMER: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

