
    POTASSIUM PHOSPHATE TRIBASIC ANHYDROUS ACS

Safety Data Sheet 
Emergency Telephone Number: CHEMTREC 1-800-424-9300               For Information call: DUDLEY 732-886-3100        

      DUDLEY CORPORATION

           
       125 Kenyon Drive

      



          
       Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT: Syn: TriPotassium phosphate. CAS No.:   7778-53-2.   Weight: 212.27     Formula: K3O4P.
2. HAZARDS: GHS:  Serious Eye Damage (Cat 1), Specific target organ toxicity – single exposure (Cat 3) Respiratory system 
Pictogram:



                [image: image1.png]www onsafilines.com

Caution



  [image: image2.png]Corrosive








Signal Word:         DANGER

Hazard statement(s):
H318 Causes serious eye damage. H335: May cause respiratory irritation.

Precautionary statement(s): P261: Avoid breathing dust/fume/gas/mist/vapors/spray.
P330: Rinse mouth.
P271: Use only outdoors or in well-ventilated area. P280 Wear PPE 
P405: Store locked up.

P304 + P340 IF INHALED: Remove person to fresh air and keep comfortable for breathing, call a POISON CENTER or doctor/ physician if unwell.  P305 +P351 + P338: If in eyes, rinse cautiously for several minutes; call poison center/doctor

P310: Remove contacts if easy to do so, P403 +P233: Store in well-ventilated area. Keep container tightly closed. 

P501: Dispose of contents/container to an approved waste disposal plant.
HMIS Classification: Health hazard  1, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  1,  Fire     0, Reactivity hazard   0

Potential Health: Eyes: Causes irritation, redness and pain.  Crystals may be abrasive.

Skin:  irritation to skin.  Ingestion: May be harmful if swallowed.  Inhalation: Causes irritation to the respiratory tract
2.3: Other Hazards: none.  
3. COMPOSITION / INFO: 

Chemical




CAS No.

Percent

Hazardous
Potassium Phosphate tribasic ACS                            7778-53-2
95-100%                    No
4. FIRST AID:  Eyes: Flush eyes with plenty of water for 15 min.  Skin:  flush skin with plenty of water. Ingestion: If victim is conscious and alert, give 2-4 cupfuls water or milk to drink.  Do Not induce vomiting.  NEVER give anything by mouth to an unconscious person.  Inhalation: Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. 

5. FIRE FIGHTING: Info: Wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH, & full PPE. Irritating & highly toxic gases may be generated. Special Hazards: Oxide of Phosphorous, Potassium oxides. Extinguishing:  Water spray, dry chemical, carbon dioxide or alcohol-resistant foam.
6. ACCIDENTAL RELEASE: Info:  Use proper PPE as indicated in Section 8.  Shut off all sources of ignition. Evacuate area. Spills/Leaks: Vacuum or sweep up material and place into a suitable disposal container.  Use non-sparking tools and equipment.  Wash spill site after material pick-up is complete. Do not let product enter drains. 

7. HANDLING & STORAGE: Handling:  Wash thoroughly after handling.  Use with adequate ventilation. Do not get in eyes, on skin, on clothing.  Avoid prolonged or repeated exposure. Storage:  Keep tightly closed.  Keep away from combustible materials, heat, sparks, and open flame.  Store in a cool dry, well ventilated area away from incompatible substances. Hygroscopic.
8. EXPOSURE / PPE:  Facilities storing or utilizing this material should be equipped with an eyewash facility and safety shower.  Use adequate ventilation to keep airborne concentrations lows.
Chemical Name



ACGIH

NIOSH

OSHA – Final/Vacated PEL’s
Potassium Phosphate tribasic


None listed
None listed
None listed

PPE: Eyes:  Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN 166. Safety goggles. Skin:  Wear appropriate protective gloves to prevent skin exposure.  0.11mm thick nitrile. Clothing: Wear appropriate PPE, including boots, lab coat, apron or coveralls, to prevent contract. Respirators:  Follow the OSHA respirator regulation found in 29 CFR 1910.134 or European Standard EN 149.  Always use a NIOSH or European Standard EN 149 approved respirator when necessary. Hygiene: Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.
9. PHYSICAL & CHEM: Appearance: White powder. Solubility: 62.4 g/l at 20°C Density: 2.564  g/cm3 at 25˚C (77˚F)  pH: 12 at ca.10g/l at 20˚C (68˚F)  
10. STABILITY & REACTIVITY: Stability: Stable under ordinary conditions of use/storage. 

Avoid:  Exposure to moisture.  Incompatibilities: Strong Oxidizing Agents Hazardous Decomp.: see section 5
11. TOXICOLOGICAL INFO:  RTECS: TC8450000   LD50 Oral Rat 4,500 mg/kg  LD50 4,640mg/kg Dermal: Rabbit- Prolonged skin contact may cause skin irritation and/or dermatitis. 
Carcinogenicity:  Not listed by ACGIH, NIOSH, NTP, IARC, or OSHA. 

12. ECOLOGICAL: Toxicity: LC50 –Rainbow trout 100mg/l-96h   EC50: Water Flea >100mg/l-48h   ErC50: Green Algae >100 mg/l – 72h    Sludge: >1,000 mg/l – 3h
13. DISPOSAL CONSIDERATIONS Chemical waste generators must determine whether a discarded chemical is classified as hazardous waste.  US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3.  Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. Contact a licensed professional waste disposal service to dispose of this material. 
RCRA P-Series:  None listed.
RCRA U-Series:  None listed.
14. TRANSPORT REGULATIONS: Not Regulated
15.REGULATORY INFO:  Section 311/312:Acute Health Hazard
SARA Section 302 Extremely Hazardous Substances: None of the chemicals in this product have a TPQ/RQ.

Section 313:  No chemicals are reportable under Section 313.

STATE: is not present on state lists from CA, MA, MN, FL.
NJ & PA Right To Know: Tripotassium orthophosphate  CAS#7778-53-2 
California No Significant Risk Level:  None of the chemicals in this product are listed.

EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives

Hazardous Symbols:  C
WGK (Water Danger/Protection)  CAS#7778-53-2:  3

Canada – DSL/NDSL CAS#7778-53-2is listed on Canada’s DSL List.  CAS#7778-53-2is not listed on Canada’s NDSL List.

Canada – WHMIS  Classification: E
16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

