
     SILVER METALLIC
Safety Data Sheet

Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       

      DUDLEY CORPORATION
           
     125 Kenyon Drive

       



         
     Lakewood, New Jersey 08701

Effective Date: January 1, 2025
1. PRODUCT ID: CAS No.: 7440-22-4 E-C No.:231-131-3
Weight:  107.87g/mol 
Formula: Ag
2. HAZARDS ID: GHS Class:  Acute aquatic Toxicity (Category 1)  Chronic aquatic toxicity (Category 1) Skin irritation (Cat 2) Serious eye Damage (Caat 1)
Pictogram:



        [image: image1.png]Corrosive




[image: image2.png]www onsafilines.com

Hazardous to the Aquatic
Environment




Signal Word:
DANGER
Hazard Statements:
H410: Very toxic to aquatic life with long lasting effects H318: Causes Serious eye damage
H315: Causes Skin irritation

Precautionary statements: P273: Avoid release into the environment.  P280: Wear protective gloves / eye and face protection. P302 + P352: IF ON SKIN: wash with plenty of soap and water. P305 +P351 + P338 +P310: Remove contact lenses, if present and easy to do. P332 + P313: IF skin irritation occurs: Get medical attention. P362: Take off contaminated clothing and wash before reuse. P391: Collect spillage  P501: Dispose of content/container to an approved waste disposal plant. P264: Wash skin thoroughly after handling. 
HMIS Classification: Health hazard  0, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  0, Fire     0, Reactivity hazard   0

Potential Health Effects: Eye: Risk of eye irritation
Skin: Maybe Harmful in contact with skin. 

Ingestion: Harmful if swallowed. 

Inhalation: Harmful by inhalation. 
2.3: Other Hazards: none
 3. COMPOSITION / INFO ON INGREDIENTS: 
Chemical Name


 CAS No.
 
Percent


Silver metallic

                7440-22-4
100%                  

4. FIRST AID: Eyes: rinse with plenty of water Skin:. Rinse with plenty of soap & water Ingestion: Do not induce vomiting w/o medical advice. Rinse with water. Inhalation: Move to fresh air.  Physician: Treat symptomatically & supportively. 

5. FIRE FIGHTING: Info: wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH & full PPE. Irritating and highly toxic gases may be generated by thermal decomp/combustion. Non-combustible, substances itself does not burn but may decompose upon heating to produce irritating, corrosive and/or toxic fumes. Extinguishing: Use water spray, dry chemical, carbon dioxide, or alcohol-resistant foam. Special Hazards: silver oxides 

6. ACCIDENTAL RELEASE: Info: Use proper PPE as indicated in Section 8. Spills/Leaks: Vacuum or sweep up material and place into a suitable disposal container. Avoid runoff into storm sewers and ditches which lead to waterways. Clean up spills immediately, observing precautions in the Protective Equipment Section. Avoid generating dusty conditions. Provide ventilation. 

7. HANDLING & STORAGE: Handling: Wash thoroughly after handling. Remove contaminated clothing & wash before reuse. Use with adequate ventilation. Minimize dust generation/accumulation. Avoid contact with eyes, skin, and clothing. Keep container tightly closed. Avoid ingestion and inhalation. Store protected from light. Storage: Store in a tightly closed container. Store in a cool, dry, well-ventilated area away from incompatibles. Store protected from light. Air SENSITIVE
8. EXPOSURE CONTROLS / PPE: Controls: Facilities storing or utilizing this material should be equipped with an eyewash facility and a safety shower. Use adequate ventilation to keep airborne concentrations low. 
Chemical Name             ACGIH                 NIOSH                        OSHA - Final / Vacated PELs

Silver Iodide                    0.01mg/m3            0.01mg/m3            0.01mg/m3    
PPE : Eyes: Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. Skin: Wear appropriate protective gloves to prevent skin exposure.  0.11mm thick nitrile. Clothing: Wear appropriate protective clothing to prevent skin exposure. 

Respirators: A respiratory protection program that meets OSHA’s 29 CFR 1910.134 and ANSI Z88.2 requirements or European Standard EN 149 must be followed whenever workplace conditions warrant a respirator’s use. 

9. PHYSICAL & CHEM: powder Relative Density: 10.49 g/ml Boiling Pt: 2,212°C    Melting Pt: 960°C
10. STABILITY & REACTIVITY: Stability: Stable under normal temp and pressures. Avoid: High temp, light, dust generation. Incompatibilities: Oxygen, Strong oxidizing agents, strong bases Hazardous Decomp: see section 5
11. TOXICOLOGICAL INFO: RTECS#: VW4450000. LD50: Oral rat >2,000mg/kg Dermal: >2,000mg/kg Inhalation – 4h- >5.16mg/l   Carcinogenicity: Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA.   Tumors at site/application
12. ECOLOGICAL INFO:  Very Toxic to aquatic life with long lasting effects. Fathead minnow: 0.001mg/l – 96h, Water Flea – 0.0022mg/l – 48h, green algae – 0.00054mg/l – 24h
13. DISPOSAL CONSIDERATIONS: Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. US EPA guidelines for the classification determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must consult state and local hazardous waste regulations to ensure complete and accurate classification. RCRA P-Series: None listed. RCRA U-Series: None listed. 

14. TRANSPORT REGULATIONS: 
DOT: UN 3077, Environmentally hazardous substance, solid, n.o.s., (colloidal Silver), 9, III  RQ:1lb
IMDG: UN 3077, ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S., (colloidal Silver), 9, III  
Marine Pollutant

IATA:  UN 3077, Environmentally hazardous substance, solid, n.o.s., (colloidal Silver), 9, III  
EHS Mark require for single & combo pack containing >5 kg for solids
15. REGULATORY INFO: SARA 311/312: None
Section 302 None of the chemicals in this product have a TPQ/RQ
Section 313 This material contains Silver, which is subject to the reporting requirements of Section 313 of SARA Title III and 40 CFR Part 373 Silver CAS#7440-22-4  Rev. 04/24/1993. 

STATE CAS# 7440-22-4  is not present on state lists from CA, MN, MA, FL. 

Pa /NJ Right to Know:
Silver  CAS#7440-22-4  Rev. 07/1/2007
California Prop.65 components: None of the chemicals are known to cause cancer, birth defects, or any other reproductive harm.

EUROPEAN/INTERNATIONAL REGULATIONS European Labeling in Accordance with EC Directives 

WGK (Water Danger/Protection) CAS# 7783-96-2: 1 

Canada - DSL/NDSL CAS# 7783-96-2 is listed on Canada’s DSL List. 

Canada - WHMIS This product has a WHMIS classification of D2A. 

Canadian Ingredient Disclosure List CAS# 7783-96-2 (listed as Silver) is not listed on the Canadian Ingredient Disclosure List. 

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 2 of 2

