
     SODIUM FLUORIDE
Safety Data Sheet

Emergency Telephone Number: CHEMTREC 1-800-424-9300       For Information call: DUDLEY 732-886-3100       

      DUDLEY CORPORATION
           
       125 Kenyon Drive

       



         
       Lakewood, New Jersey 08701

Effective Date: January 1, 2025


1. PRODUCT ID: CAS No.: 7681-49-4       E-C No.:231-667-8   Weight:  41.99g/mol  Formula: FNa

2. HAZARDS ID: GHS:  Acute toxicity, Oral (Cat 3) Skin irritation (Cat 2)
Eye irritation (Cat 2A) skin irritation (Cat 2)


 Pictogram: 



         [image: image1.png]www onsafilines.com

Acute Toxicity



          [image: image2.emf]
Signal Word:         DANGER

HAZARD STATEMENT(S): H301: Toxic if swallowed   H319: Causes serious eye irritation.  H315: Causes skin irritation

PRECAUTIONARY STATEMENT(S): P264: Wash skin thoroughly after handling. P270: Do not drink, eat, smoke, when using this product. P405: Store locked up. P280: Wear protective gloves/eye & face protection.  P302 + P352: If on skin gently wash with plenty of soap & water. P301 + P310+ P330: If swallowed: Immediately Call Poison Center or doctor/physician if you feel unwell, Rinse mouth. P305 + P351 + P338: If in Eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present & easy to do so. Continue rinsing. P362: Take off contaminated clothing and wash before reuse. P332 + P313: If skin irritation occurs: consult physician. P337 + P313: If eye irritation occurs: consult physician. P501: Dispose of contents/container to an approved waste disposal plant.

HMIS Classification: Health hazard  2, Flammability   0,  Physical hazards   0




NFPA Rating: Health hazard  2, Fire     0, Reactivity hazard   0

Potential Effects: Eye: Risk of serious damage to eyes.
Skin: Harmful in contact with skin. 

Ingestion: Harmful if swallowed. 

Inhalation: Harmful by inhalation. 
2.3: Other Hazards: none
3. COMPOSITION / INFO: 
Chemical



 CAS No.
 
Percent


Sodium fluoride

                  7681-49-4                 90-100

4. FIRST AID: Inhalation: Move to fresh air.  Consult a physician.  Eyes: rinse immediately for 15 min, with plenty of water and seek medical advice. Skin. Rinse with plenty of soap & water and seek medical advice. Ingestion: Do not induce vomiting without medical advice. Rinse with water. Consult physician. Physician: Treat symptomatically & supportively.

5. FIRE FIGHTING: Fire: Not considered to be a fire or explosion hazard. Extinguishing: Dry powder Hazards:  Hydrogen fluoride, sodium oxides. Info: wear full PPE and NIOSH-approved self-contained breathing apparatus with full face-piece operated in the pressure demand or other positive pressure mode.

6. ACCIDENTAL RELEASE:  Ventilate area of leak or spill. Wear appropriate PPE as specified in Section 8. Evacuate personnel to safe areas. Avoid breathing dust, vapors, gas, or mist and avoid making dust formation.   Spills: Pick up and place in a suitable container for reclamation or disposal, using a method that does not generate dust. Prevent further leakage or spillage if safe to do so. Do let product enter drains, discharge to environment must be avoided. 
7. HANDLING & STORAGE:  Keep in a tightly closed container, stored in a cool, dry, ventilated area. Moisture Sensitive; Keep in a Dry place. Protect against physical damage. Containers of this material may be hazardous when empty since they retain product residues (dusts, solids); observe all warnings and precautions listed for the product.

8. EXPOSURE CONTROLS:  Facilities storing or utilizing this material should be equipped with an eyewash facility and safety shower.  Use adequate ventilation to keep airborne concentrations low.
Chemical 
      ACGIH
                 

NIOSH

 OSHA – Final / Vacated PEL’s

Sodium Fluoride
TWA 2.5mg/m3 (TLV)  
 
2.5mg/m3
              2.5mg/m3 TWA



Fluoride 3-10mg/g, 2-3mg/l (BEI)
PPE: Eyes:  Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA’s eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166. Skin:  Wear appropriate protective gloves to prevent skin exposure. 0.11mm thick nitrile. Clothing: Wear appropriate protective clothing to prevent skin exposure.    Respirators:  Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149.  Use a NIOSH/MSHA or European Standard EN149 approved respirator if exposure limits are exceeded or if irritation or other symptoms are experienced. Hygiene measures: Follow general industrial hygiene and health practices. 

9. PHYSICAL & CHEM: Fine, white or slightly yellow powder. Melting Pt: 993˚C (1,819˚F) Relative Density: 2.780 g/cm3 Vapor Pressure: 1.9 hPa (1.4 mmHg)
10. STABILITY & REACTIVITY: Stability: Stable under ordinary conditions of use and storage. Haz Decomp: None Incompatibilities: Strong acids & glass Avoid: Heat, incompatibles, moisture
11. TOXICOLOGICAL INFO: RTECS: WB0350000
LD50: Intravenous rat: 26mg/kg: weight loss
Oral rat LD50: 148.5 mg/kg;
Irritating to skin.  Rabbit eye: 24h – moderate eye irritation 
Oral TDLo: Human 0.214 mg/kg; 7 mg/kg ~ Gastrointestinal changes, Eye Ptosis, Cyanosis 

Carcinogenicity: Not classifiable as to its carcinogenicity based on its IARC, ACGIH, NTP, or EPA Classification. 

IARC: 3-Group3: not classifiable as to its carcinogenicity to Humans (Sodium Fluoride).

12. ECOLOGICAL INFO: Harmful to aquatic life.
Environmental Toxicity: Toxicity to Fish ~ mortality NOEC – Cyprinodon variegates (sheepshead minnow)-500mg/l – 96 h; LC50 – Oncorhynchus mykiss (Rainbow trout) – 200 mg/l – 96h
Toxicity to daphnia & aquatic invertebrates: EC50 Daphnia magna (water flea) – 98mg/l – 48h

Bio-accumulative potential: Salma trutta – 10d – Bioconcentration factor (BCF) 2.3

13. DISPOSAL CONSIDERATIONS: Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste disposal facility. Processing, use, or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Burn in a chemical incinerator equipped with an afterburner and scrubber. Dispose of container and unused contents in accordance with federal, state, and local requirements.

14. TRANSPORT REGULATIONS: Reportable Quantity (RQ):  1000 lbs

DOT (US)                   UN 1690, Sodium fluoride, solid, 6.1, III, ERG 154



IMDG

UN 1690, SODIUM FLUORIDE, SOLID,, 6.1, III, ERG 154

EMS-No: F-A, S-A

IATA

UN 1690, Sodium fluoride, solid, 6.1, III, P.I. 

15. REGULATORY INFO: SARA 311/312: Acute Health Hazard, Chronic Health Hazard.
SARA 302: No chemicals in this material are subject to the reporting requirements of SARA Title III, Section 302.

SARA 313: This material does not contain any chemical components with known CAS # that exceed threshold (De Minimis) reporting levels established by SARA Title III, Section 313. 
STATE CAS# 7681-49-9 is not present on state lists from CA, MN, FL. 

Mass Right To Know components: Sodium Fluoride CAS 7681-49-4 Revision Date 2007-03-01

NJ Right To Know components: Sodium Fluoride CAS 7681-49-4 Revision Date 2007-03-01

Pa Right To Know components: Sodium Fluoride CAS 7681-49-4 Revision Date 2007-03-01

California Prop.65 Components: This product does not contain any chemicals known to State of CA to 
cause cancer, birth defects, or any other reproductive harm. 

16. DISCLAIMER INFO: DUDLEY CORPORATION provides the information herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this.  Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. DUDLEY CORPORATION makes no representations or warranties, either expressed or implied, including without limitation and warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers. Accordingly, DUDLEY CORPORATION will not be responsible for damages resulting from the use of or reliance upon this information.   

Page 1 of 1

